# Book 02 — Network Modernization

AAN module: from mainframe to application-aware modernization (SD-WAN · SASE · the four principles)

Federal AAN Training Program — companion material for the federal AAN book series; not part of UIAO.

Source: Book_02 Network Modernization <https://github.com/WhalerMike/uiao/blob/main/inbox/Application%20Aware%20Networking/Book_02_Federal_Application_Aware_Networking_Network_Modernization.qmd> · Evidence slot: 3 — network (KSI-CNA, SC-8 / CA-7) · Track modules: A3, B2

## Scope

A narrative history in four acts of how federal IT lost its single source of truth — mainframe centralization, 1990s decentralization into regional silos, security stacking that broke session-based Kerberos, and the multi-cloud breaking point — pivoting on one sentence: Active Directory conflated the directory (truth) with the domain controller (in-path enforcer). Thesis: restore SSOT, adopt application-aware networking, move to token-based identity, and govern actively — physics and zero-trust mandates make the perimeter/session model unrecoverable.

## Learning objectives

Compute the speed-of-light propagation floor (e.g., Portland–Washington ≈ 5,400 km fiber ≈ 27 ms one-way) and explain why bandwidth cannot fix latency.

Explain the two independent session killers — latency exceeding protocol timeouts, and asymmetric routing breaking stateful inspection — and how SD-WAN overlay tunnels remove asymmetry.

Contrast packet-level (five-tuple) with application-aware enforcement (user, device posture, app, data, context) and name the NIST controls only the latter satisfies.

Articulate why DIA alone satisfies zero NIST controls and which SASE/SD-WAN components close AC-4, SC-7, SC-8, SI-3, SI-4, AU-2/12, CA-7.

Compare transports (MPLS hairpin, DIA, 5G MEC/CUPS, LEO satellite) and explain active-active multipath, FEC, and resequencing.

Place governance postures on the CISA ZTMM actuation ladder and identify gated actuation as the AO-signable federal standard.

## Key concepts

Truth/enforcer conflation in Active Directory (the "one sentence").

The physics floor: fiber at ~200,000 km/s; LEO at ~550 km altitude (20–40 ms) vs. the old GEO 238 ms ruling.

"DIA is a pipe" — DIA enables compliance; AAN/SASE achieve it.

The Four Principles of Application-Aware Modernization: SSOT, AAN, token identity, active governance.

Active Governance actuation ladder: Traditional → Initial → Advanced (gated) → Optimal.

FedRAMP 20x continuous machine-readable KSI evidence replacing periodic assessment.

## Reading sequence

Argument-shaped, not implementation-shaped: prologue (mainframe = truth + enforcement in one box) → Act 1 (decentralization distributes truth) → Act 2 (security stacking + physics break sessions) → Act 3 (multi-cloud fractures identity) → physics interlude (DIA, LEO, 5G, SD-WAN multipath) → Act 4 (the four principles mapped to federal mandates and FedRAMP 20x).

## Compliance hooks

AC-4, SC-7, SC-8, SI-3, SI-4, AU-2/AU-12, CA-7, CM-8, SI-12, SC-20/SC-21. Frameworks: NIST SP 800-207, SP 800-63, SP 800-37 Rev 2; EO 14028; OMB M-22-09 §3/§4; BOD 25-01 (SCuBA); TIC 3.0 Use Cases E/F; CISA ZTMM v2.0; FedRAMP 20x Consolidated Rules.

## Labs

Physics calculator: compute propagation floors for agency city pairs over fiber vs. LEO and compare against measured MPLS-hairpin latency.

Asymmetric-routing demo: two-path lab with a stateful firewall on one path; observe SYN-ACK drops, then tunnel both directions through one overlay endpoint.

Policy translation: rewrite five-tuple firewall rules as application-aware policies and map each to the NIST control it newly satisfies.

## Pitfalls the book calls out

Concluding "DIA solved the network problem" — connectivity without the control stack is not compliance.

Treating VPN as zero trust — a tunnel extends network-wide trust to any compromised endpoint inside it.

Deploying tokens atop fragmented directory trees without SSOT.

Letting the governance plane become an in-path enforcer, or automating high-blast-radius actions that must remain human-gated.

## Vendor training

Cisco U. / NetAcad, Zscaler, Netskope, Fortinet / Palo Alto, AZ-700, AWS Skill Builder.

Previous: Book 01 · Next: Book 03 — Federal Telecommunications Modernization