# Book 03 — Federal Telecommunications Modernization

AAN module: cloud voice on Amazon Connect and Teams Phone over one SD-WAN investment

Federal AAN Training Program — companion material for the federal AAN book series; not part of UIAO.

Source: Book_03 Federal Telecommunications Modernization <https://github.com/WhalerMike/uiao/blob/main/inbox/Application%20Aware%20Networking/Book_03_Federal_Application_Aware_Networking_Federal_Telecommunications_Modernization.qmd> · Evidence slot: 1 — identity (KSI-IAM, IA-2 / AC-2) · Track modules: A1, B2

## Scope

The agency's two cloud voice platforms — the N8NN citizen contact center on Amazon Connect (AWS commercial .com, by authorized exception) and internal telephony on Microsoft Teams Phone in GCC Moderate — share identical network requirements (<75 ms one-way, <30 ms jitter, <1% loss, DSCP EF). One SD-WAN/AAN modernization investment therefore enables both; voice is the canary that exposes network problems other applications hide.

## Learning objectives

State the ITU-T G.114 150 ms one-way delay ceiling and derive the ~75 ms network budget after codec and jitter-buffer overhead.

Map network conditions to MOS via the ITU-T G.107 E-model and defend the ≥4.0 MOS floor.

Explain the Amazon Connect .com authorized exception, its SSP SA-9 boundary documentation, and ATO-renewal maintenance obligations.

Diagnose asymmetric routing as a stateful-firewall session killer and explain why SD-WAN overlay eliminates it structurally.

Plan Teams Phone Direct Routing (SBC selection, FedRAMP-authorized cloud SBC) and E911 compliance under Kari's Law and Ray Baum's Act.

Size DSCP EF capacity for hybrid agents (max concurrent hybrid agents × 200 Kbps).

## Key concepts

MPLS hairpin vs. local breakout (DIA), with the Portland → Washington → AWS us-west-2 worked example.

AWS .us vs. .com vs. GovCloud federal delivery models.

WebRTC/RTP/SRTP requirements; DSCP EF (46) steering; FEC at ≥0.5% loss.

Hybrid agent = two simultaneous DSCP EF streams (Connect + Teams Phone).

"DIA enables; SASE achieves" — the pipe vs. the control stack.

Direct Routing SBC options (on-prem vs. cloud) and E911 LIS/ELIN.

## Implementation sequence

Phase 1 (months 1–3): SD-WAN, local breakout, DSCP EF steering, FEC; measure and document G.114 compliance per location — a hard gate before any migration.

Phase 2 (months 2–5): Amazon Connect — exception confirmation, contact flows, Entra ID federation, pilot with MOS measurement, S3 recording pipeline validation.

Phase 3 (months 4–8): Teams Phone — SBC deployment, per-location E911 testing, PBX number inventory, geographic porting tranches with parallel PBX operation.

Phase 4 (months 6+): continuous monitoring — MOS ≥4.0 SLA alerting, SIEM integration, ATO exception review, 20x KSI alignment.

## Compliance hooks

AC-4, SC-7, SC-8, SI-3, SI-4, AU-12, CA-7, SA-9, AU-9, SC-28, IA-8. Also ITU-T G.114/G.107, RFC 3246/3550/4594, TIC 3.0 Use Case E, OMB M-22-09, CISA ZTMM v2.0, Kari's Law and Ray Baum's Act (47 CFR Part 9), Privacy Act.

## Labs

Measure one-way latency, jitter, and loss to AWS us-west-2 and Microsoft 365 endpoints; compute the G.114 verdict for hairpin vs. direct paths.

SD-WAN policy lab: DSCP EF → lowest-latency path, HTTPS → highest-throughput path; verify EF markings are honored, not remarked.

Configure and test a Teams emergency-calling policy with LIS dispatchable location, documenting the 911 test.

## Pitfalls the book calls out

Migrating any office before G.114 validation — post-migration voice failures cost dramatically more.

Treating DIA alone as compliance: it fixes latency but satisfies zero NIST controls without the SASE stack.

Asymmetric routing misdiagnosed as "SBC misconfiguration."

Assuming the .com exception is permanent — it requires re-evaluation each cycle; cloud SBCs need their own FedRAMP authorization.

## Vendor training

SC-300, AWS Skill Builder (Amazon Connect), Zscaler / Netskope.

Previous: Book 02 · Next: Book 04 — SQL Server Authentication Modernization